
ZBJ-PZ-100T-1500T系列单动薄板拉伸液压机

                ZBJ-PZ-100T-1500T series Hydraulic Stamping Press
[image: image1.jpg]



● 适用于所有拉伸、弯曲、成形、冲裁落料、翻边等各种工艺。

Sheet metal stretching, blanking, stamping and all stretch flange forming process, but also for correction, press-fit technology, widely used in the following industries:
● 计算机优化结构设计，四柱式结构简单、经济、实用；框架式结构刚性好、精度高、抗偏载能力强。
● 液压控制采用插装式集成系统，动作可靠，使用寿命长，液压冲击小，减少了连接管路与泄漏点。
● 采用PLC控制的电气系统，结构紧凑、工作灵敏、可靠、柔性好。
● 通过操作面板选择，不仅可实现定程，定压两种成形工艺，而且可实现有垫、无垫、顶出三种工艺工作循环。
另外可以选配前后知道就行辊道和机械手。

Application:
  

Sheet metal stretching, blanking, stamping and all stretch flange forming process, but also for correction, press-fit technology, widely used in the following industries:
 

Auto parts: body panels, brake, fuel tank, chassis, axle housing, bumper
 

Kitchen appliances: sink, a pot, stainless steel appliances, and other containers.
 

Other: tractors, motorcycles, aerospace, aviation, chemicals, instruments, machine tools
Advantages:
 

1.Simple 4-column structure and reliable operating system 
2.With movable working table 
3.Easy and practical to change die 
4.Uses PLC control to make sure that operating sensitive and reliable
5.Mechanical, electrical and hydraulic integration
 

Product Description:
 

The hydraulic press use optimized design of computer with international advanced level. The frame structure, rigid, high precision, strong resistance to side load capacity. Corners directions with guide,high guiding precision New fuel tank sealing components, high reliability, long life.
 

The hydraulic press have fast working speed,improve the production efficiency. The working pressure and stroke can be adjusted according to process requirements, easy to operate.
 

Our hydraulic have low noise which can replace the mechanical press.
 

Using non-contact displacement sensor or switch, improve the reliability and accuracy of stroke control.
 

Mechanical, electrical, hydraulic integration of safety devices, security is superior.

 

TECHNICAL SPECIFICATIONS:
	Item
名称
	Unit单位
	  ZBJ-100T
	ZBJ-200T
	ZBJ-315T
	ZBJ-400T
	ZBJ-500T
	ZBJ-630T
	ZBJ-800T
	ZBJ-1000T

	Nominal force

公称力
	KN
	1000
	2000
	3150
	4000
	5000
	6300
	8000
	10000

	Return force

回程力
	KN
	160
	240
	600
	500
	900
	850
	1000
	1350

	Slide stroke
	KN
	510
	710
	800
	800
	900
	900
	1000
	1000

	Open height打开高度
	mm
	800
	1120
	1250
	1250
	1500
	1500
	1800
	1800

	Hydraulic cushion nominal force拉伸力
	kn
	400
	630
	1250
	1250
	2000
	1250*2
	1600*2
	2000*2

	Hydraulic cushion return force拉伸滑块行程
	kn
	147
	250
	360
	360
	780
	360*2
	600*2
	580*2

	Hydraulic cushion stroke

压边滑块行程
	mm
	200
	250
	300
	300
	350
	350
	400
	400

	Slide speed

拉伸滑块速度
	Down

下降
	mm / S
	100
	150
	100
	130
	120
	130
	150
	130

	
	Work

工作
	mm / S
	7-15
	5-12
	5-12
	4-10
	7-15
	5-12
	5-12
	5-12

	
	Return 回程
	mm / S
	85
	95
	60
	70
	80
	85
	95
	85

	Hydraulic cushion speed

压边滑块速度
	Up

上
	mm / S
	35
	38
	30
	30
	37
	30
	30
	30

	
	Return

回程
	mm / S
	100
	95
	100
	100
	95
	105
	85
	105

	Working table工作台面尺寸
	L-R

左右
	mm
	690
	1000
	1260
	1250
	1400
	1600
	2200
	2500

	Effective size有效工作台面尺寸
	Front-back前后
	mm
	630
	940
	1160
	1250
	1400
	1600
	1600
	1800

	Motor Power

电机功率
	KW
	7.5
	15
	22
	22
	44
	44
	60
	66


 

                           ZBJ100-300系列橡塑液压机

[image: image2.jpg]-





	参数名称Parameter
	单位Unit
	ZBJ-63
	ZBJ-100
	ZBJ-160
	ZBJ-200
	ZBJ-250
	ZBJ-300

	公称压力Nominal pressure
	KN
	630
	1000
	1600
	2000
	2500
	3000

	滑块行程Stroke of Slide block
	mm
	500
	600
	700
	700
	800
	800

	滑块下平面距工作台的最大距离

Max distance from bottom surface of slide block to worktable
	mm
	800
	800
	800
	1120
	1300
	1300

	工作台尺寸（左右×前后）Worktable size(Right to left ×Front to back )
	mm
	580×500
	720×580
	900×730
	900×900
	950×900
	1000×1000

	液体最大工作压力Max work pressure of liquid
	Mpa
	25
	25
	25
	25
	25
	25

	滑块下行速度Descent speed of slide block
	mm/s
	50
	50
	50
	60
	60
	60

	滑块回程速度Return speed of slide block
	mm/s
	60
	60
	60
	50
	50
	40

	外形尺寸（长×宽×高）Overall Dimensions (length×width × height)
	mm
	780×500
×2900
	950×580 ×2900
	1180×780
×3500
	1340×940
×3900
	1600×1200 ×4000
	1800×1400
×4000

	电机功率Electric Power
	kw
	5.5
	5.5
	7.5
	11.5
	15
	15

	重量Weight
	kg
	2800
	3500
	5600
	7800
	8200
	10000


本机广泛应用于热固性塑料、密胺餐具类、胶木、电器绝缘材料（DMC\SMC）的热压成型
 
该系列产品液压系统由本厂独立开发设计，性能稳定可靠，操作简单安全。
 
电器系统采用PLC可编程序控制，一般工艺程序为：滑块快下—减速慢下---压制（可根据工艺要求来设定排气次数和排气时间）---保压（保压时电机停止工作）--（保压时间到）泄压—滑块慢回—快回至上限位—顶出缸慢顶—顶出缸快顶至限位—取胚—延时—顶出缸退回——手工加料，整个工艺顺利完成。
Application: Thermosetting plastic, melamine tableware, bakelite, electrical insulating materials (DMC \ SMC) Etc.

 2.Advantages:
 1. Simple and convenient structure, flexible operation,commodious space. The cylinder and plunger are made up of high–quality materials. 
 2. Hydraulic station and electric cabinet are Located in both sides of the main engine.
 3. Conveniently operated.
 4. Without oil spilling or off pressure.
 5. Excellent reputation.
3.Making process

	Mixing
	Pour the rubber granules and coloring pigment into the rubber mixing machine,
with some adhesie,then begin stiring. It will take 4-6 mins to be done.    

	Weighing
	Weigh the stirred rubber powder according to the floor tiles thickness you want 
to make.

	Paving and Pressing
	Pour the needed quantity rubber powder into the mould, and float it by trowel
On the bottom ,can be with some coarse and black  rubber powder, but on the 
surface should be with fine and colored powder, it can ensure your tiles quality. 

	Vulcanization 
Moulding
	Put the mould on the vulcanizer layer, generally , it take 3/8 mins to make the 
tiles, time difference basedon the thickness of the tiles you ready to make.After 
finished, it will automaticlly separated,then put it somewhere to dry.    

	Inspecting
	Inspect the tiles' surface . If no aberration, no flaw, then it is of high quality 
products



